KATEST26 – TF02

Date: 8 April 2026

Objective: Wind calibration maneuver flight. Rodi maneuvers followed by Lenschow maneuvers at 20k ft AMSL, then repeated at 14k ft AMSL.

Notes:

- PBLH was ~12,500-13,000 ft AMSL

- Winds were westerly

- Flight was conducted near Scottsbluff, NE

- Probe heats OFF entire flight

- Conditions were clear at flight level – some high cirrostratus above at points (~22k ft), and some popcorn cumulus below us (~11k ft) at points

Flight notes (times in UTC):

1605 Taxi

1610 Wheels up

- Pitch and roll seem correct after Applanix direction adjustments before flight

1617 Pilot rudder test

1633-1642 Finding nice clear spot for maneuvers at 20k ft

1647 CCW Rodi yaw maneuver

- Heading: W

1650 CW Rodi Yaw maneuver

1653 Done w/ Rodi yaw maneuvers

- Switching to N heading

1655 CCW Rodi speed maneuver

- 1656 reached minimum aircraft speed (120 kts), speeding back up to 180 kts

- 1657 Done

1657 CW Rodi speed maneuver

- 1659 reached minimum aircraft speed (120 kts), speeding back up to 180 kts

1700 Done with Rodi maneuvers

- Turning to start Lenschow maneuvers

- Winds out of ~270

1703 Aligned with wind, starting deceleration to 120 kts

1707 Accelerating back to 170 kts

1707 Lenschow maneuver 1 done

- 180 to get back into flight block

1711 Starting Lenschow pitch oscillations (performed 7)

- 1715 Done

1716 Start Lenschow yaw oscillations (performed 7)

- 1718 Done

1719 CW Lenschow 360 constant turn

1721 CCW Lenschow 360 constant turn

- 1725 Done

1725 Aligning heading with wind direction

1726 Start Lenschow Reciprocal heading legs

- Due E first (with wind)

1729 180 to get to reciprocal heading

1731 Lenschow reciprocal leg into wind (due West)

1734 Done with Lenschows

1735 Starting descent to 12k ft

1746 Noticed PBLH near 12.5-13k ft  going back to 14k ft

1749 Leveled out at 14k ft, starting CW Rodi yaw maneuver

- Some periodic light turbulence at this altitude

1752 CCW Rodi yaw maneuver

1755 Shifting to N heading

1757 CCW Rodi speed maneuver

- 1758 reached minimum aircraft speed (120), speeding back up

- Realized we start and were ending at a different max speed than before (170 instead of 180), so repeated this one

1759 Repeating CCW Rodi speed maneuver

- 1800 reached minimum aircraft speed (120), speeding back up to 180

1802 CW Rodi speed maneuver

- 1803 reached min aircraft speed (120), speeding back up to 180

1804 Aligning with wind to prep for Lenschow maneuver

1805 Starting acceleration/deceleration Lenschow

- Slowing down to 120

- 1807 reached min aircraft speed (120), speeding back up to 170

1811 Starting Lenschow pitch oscillations (performed 7)

- 1814 Done

1815 Lenschow yaw oscillations (performed 8)

1816 CCW Lenschow 360 constant turn

1819 CW Lenschow 360 constant turn

- 1821 light, brief turbulence

1821 Done

1822 Starting Lenschow reciprocal heading legs

- Into wind (due W) first

1825 180 to reciprocal heading

1829 Lenschows complete

- Heading back to LAR

- Ascending to 20k

1915 Wheels down