KATEST26 – TF03

Date: 8 April 2026

Mission Scientist: Anna Robertson

Objective: Repeat wind calibration maneuver flight from this morning (only at 20k ft AMSL), with probe heats on. Rodi maneuvers followed by Lenschow maneuvers at 20k ft, then spiral down to 12k ft AMSL to get vertical profile of winds and temperature.

Notes:

- Winds were westerly

- Flight was conducted near Scottsbluff, NE

- Probe heats ON entire flight

- Conditions were severe clear both above and below maneuver location

Flight notes (times in UTC):

2127 Wheels up

2129 Probe heats (Rosemount, Noseboom tip) ON

2148 Arrived in area for maneuvers at 20k ft, picking reference heading

2149 CW Rodi yaw maneuver

- 2150 Let go of bank briefly (will repeat after CCW maneuver)

2152 CCW Rodi yaw maneuver

- 2154 done

2155 Repeating CW Rodi yaw maneuver

- 2158 done

2158 Choosing new heading  N

2159 CCW Rodi speed maneuver

- Note: from here on, remembered to grab more specific time stamps (HH:MM:SS)

- 22:00:57 reached minimum aircraft speed (120 kts), accelerating back to 180 kts

22:02:10 CW Rodi speed maneuever

- 22:03:20 reached min speed (120), accelerating back to 180

- Note: noticed TROSE_raw decreased ~3 deg as aircraft speed decreased to minimum

22:04:40 done with Rodi maneuvers

- Aligning with wind for first Lenschow maneuver

22:06:30 Starting deceleration for Lenschow S&L deceleration/acceleration maneuver

- 22:08:05 reached minimum speed (120), accelerating back to 170

22:09:55 Lenschow pitch oscillation maneuver (performed 7)

22:12 Turning to stay in approved flight block

22:14:05 Lenschow yaw oscillations maneuver (performed 7)

22:16:20 CW Lenschow 360 constant turn

- 22:18:45 Caught our own wake turbulence

22:18:55 CCW Lenschow 360 constant turn

- 22:21:40 done

22:22:40 Repeating Lenschow yaw oscillations maneuver (there was a lot of roll with the first set, trying to keep wings more level while varying heading)

- 22:24:10 done

22:24:30 Starting Lenschow reciprocal heading leg – into wind

22:27:30 180 turn to reciprocal heading (adjusted reciprocal heading slightly to ensure better alignment with tail wind)

22:29:40 Lenschow reciprocal heading leg – with wind

- 22:32:50 done

22:34 Starting spiral down to 12k ft AMSL

- 500 ft/min

22:36 Ramping up to 1000 ft/min (500 too slow to meet flight time limit of ~2 hrs)

22:43 Done with spiral

22:44 Heading back to KLAR

23:29 Probe heats OFF

23:34 Wheels down